BOT Execution – Interface Status

Datapoint – “operstate”

Introduction

This KB article is in reference to the proactive incident tickets triggered by monitoring server “Logic Monitor “and is applicable to “Interface Status” alerts for network devices such as, routers, switches, access points, firewalls, etc.

Intended Audience

This document is intended for network Support Group managing network and security infrastructure related incidents globally.

Why are these “Interface Status” alerts generated by the monitoring server “Logic Monitor”?

When the collector does not receive any metrics or heartbeat from a switch (CI Name) for a continuous period of 5 minutes, Logic Monitor marks the device as ‘Idle’ and raises a Critical alert at the start of the 6th minute. This alert suppresses any other active alerts on the same host, as server-side logic designates it as Critical

Example for “The host (CI Name) is down” alerts

“OYB11255254 critical - OUYwcMUSsuncp Network Interfaces-GigaEdep1/0/100 [ID:101100] Status

Interface GigaEdep1/0/100 [ID:101100] Int_Mon *** To YUOwdJYSsghunOI *** AP on OUYwcMUSsuncp has gone down and reporting the value 2.0, placing the interface into critical state.”

PTJ90257645 -> Alert ID Generated in Service Now

OUYwcMUSsuncp -> Affected Host

GigaEdep1/0/100 -> Interface of the affected host

Int_Mon *** To YUOwdJYSsghunOI *** -> Interface Description

LogicMonitor uses SNMP polling via the device’s FQDN to collect metrics from configured devices. For interface monitoring, LogicMonitor relies on the “operstate” datapoint, which generates alerts based on the following values:

-1=Alerting Disabled, as Interface doesn't match alert enabling properties

0=Administratively Down

1=Up - Ready to pass packets,

2=Down,

3=Testing - in a test mode,

4=Unknown - status cannot be determined,

5=Dormant - interface is not actually in a condition to pass packets (i.e., it is not 'up') but is in a "pending" state, waiting for some external event.,

6=Not Present - some component is missing,

7=Lower Layer Down - down due to state of a lower-layer interface(s).

value 2.0 -> As mentioned above.

BOT troubleshooting Steps

The following matrix must be adhered to during troubleshooting

| Target Device Type | BOT Methodologies | Action - Auto Closure by BOT | Action - Assigned to Engineer |
| --- | --- | --- | --- |
| Switch A Core/Distribution Switch Naming Convetion having - "st"/"sw" | Interval - 5 Minutes , Repeat 03 times 1. Log in to Jump Server & Ping Source IP 2. Login to Source IP and Execute show int <Source Interface ID> show cdp nei <Source Interface ID> detail 3.Login to Destination Device and Execute and extract destination interface MAC-ADDR show int <Destination Device Interface ID> 4.Login to Source IP and Execute to check the MAC-ADDR is same show mac address-table interface <Source Interface ID> ¦ i <Destination Interface MAC> | After successful completion of step -04 during polling | Any Unsuccessful Step and after 15 Minutes |
| Access Switch/WLC | Interval - 10 Minutes , Repeat 03 times 1. Log in to Jump Server & Ping Source IP 2. Login to Source IP and Execute show int <Source Interface ID> show cdp nei <Source Interface ID> detail 3.Login to Destination Device and Execute and extract destination interface MAC-ADDR show int <Destination Device Interface ID> 4.Login to Source IP and Execute to check the MAC-ADDR is same show mac address-table interface <Source Interface ID> ¦ i <Destination Interface MAC> | After successful completion of step -04 during polling For WLC , Step 1 & 2 | Any Unsuccessful Step and after 30 Minutes |
| WAP | Interval - 10 Minutes , Repeat 06 times 1. Log in to Jump Server & Ping Source IP 2. Login to Source IP and Execute show int <Source Interface ID> show cdp nei <Source Interface ID> detail | After successful completion of checks during polling | Any Unsuccessful Steps and after 60 Minutes |

Additionally, for any unsuccessful closure, the following command outputs must be captured to check if any cable-fault is present in that interface or not.

Ethernet Cable Test

test cable-diagnostics tdr interface < Interface of the affected host >

show cable-diagnostics tdr interface < Interface of the affected host >

It requires a delay executing the commands.

Optical Fiber Cable Test(Mostly required when a switch or router is connected)

show int < Interface of the affected host > transceiver detail

If the connected device is an access point , it needs to be checked whether the access point is associated with the WLC

To Check this need to login WLC and Execute the command

** Attempted to retrieve AP uptime using a simple command, but due to command restrictions on the WLC this was not possible. Will continue exploring available commands and methods to obtain the information***

show ap config general <AP NAME>

Cisco AP Identifier.............................. 456

Cisco AP Name.................................... OUYwcMUSsuncpY9

Country code..................................... London

Regulatory Domain allowed by Country............. 876.12bg:-B    987.14a:-B

AP Country code.................................. London

AP Regulatory Domain............................. 876.12bg:-B    987.14a:-B

Switch Port Number .............................. 19

MAC Address...................................... f0:f1:99:11:12:ba

IP Address Configuration......................... Static IP assigned

IP Address....................................... 10.876.77.78

IP NetMask....................................... 299.299.299.0

Gateway IP Addr.................................. 10.876.77.8

Domain...........................................

Name Server......................................

NAT External IP Address.......................... None

CAPWAP Path MTU.................................. 1456

DHCP Release Override............................ Disabled

Telnet State..................................... Globally Enabled

Ssh State........................................ Globally Enabled

Cisco AP Location................................ B66

Cisco AP Floor Label............................. 0

--More-- or (q)uit

Cisco AP Group Name.............................. Dlpc78

Primary Cisco Switch Name........................ OUYwcMUSsuncp09

Primary Cisco Switch IP Address.................. 55.847.6.000

Secondary Cisco Switch Name...................... OUYwcMUSsuncp02

Secondary Cisco Switch IP Address................ 55.847.6.000

Tertiary Cisco Switch Name.......................

Tertiary Cisco Switch IP Address................. Not Configured

Administrative State ............................ ADMIN_ENABLED

Operation State ................................. REGISTERED

Mirroring Mode .................................. Disabled

AP Mode ......................................... Local

Public Safety ................................... Disabled

AP SubMode ...................................... Not Configured

Remote AP Debug ................................. Disabled

Logging trap severity level ..................... informational

KPI not configured ..............................

Logging syslog facility ......................... kern

Wireless Logging State .......................... Disabled

S/W  Version .................................... 8.0.546.0

Boot  Version ................................... 54.3.2.4

Mini IOS Version ................................ 7.0.321.45

Stats Reporting Period .......................... 180

Stats Collection Mode ........................... normal

--More-- or (q)uit

LED State........................................ Enabled

PoE Pre-Standard Switch.......................... Enabled

PoE Power Injector MAC Addr...................... Disabled

Power Type/Mode.................................. PoE/Full Power

Number Of Slots.................................. 2

AP Model......................................... AIR-LAP8989-B-P6

AP Image......................................... B7760-K93w-M

IOS Version...................................... 14.3(3)JA10$

Reset Button..................................... Enabled

AP Serial Number................................. FTX7564P6GP

AP Certificate Type.............................. Manufacture Installed

AP User Mode..................................... AUTOMATIC

AP User Name..................................... admin_1213

AP Dot1x User Mode............................... Not Configured

AP Dot1x User Name............................... Not Configured

Cisco AP system logging host..................... 399.399.399.399

AP Up Time....................................... 786 days, 00 h 33 m 59 s

AP LWAPP Up Time................................. 197 days, 13 h 27 m 50 s

Join Date and Time............................... Sat Feb 25 19:48:23 2023

Join Taken Time.................................. 0 days, 00 h 00 m 29 s

Memory Type...................................... DDR3

Memory Size...................................... 98294 KBytes

CPU Type......................................... PowerPC897exr CPU at 666Mhz, revision number 0x18B8

--More-- or (q)uit

Flash Type....................................... Onboard Flash

Flash Size....................................... 30744 KBytes

GPS Present...................................... NO

Ethernet Vlan Tag................................ Disabled

Ethernet Port Duplex............................. Auto

Ethernet Port Speed.............................. Auto

AP Link Latency.................................. Disabled

Rogue Detection.................................. Enabled

AP TCP MSS Adjust................................ Enabled

AP TCP MSS Size.................................. 1250

Capwap Prefer Mode............................... IPv4 (Global Config)

Hotspot Venue Group.............................. Unspecified

Hotspot Venue Type............................... Unspecified

DNS server IP ............................. Not Available

*************will update the Logic later (if Required) **************************